THERMISTOR-CONTROLLED LED

USING LM741 OP-AMP COMPARATOR

Temperature Sensing and Threshold Switching Circuit

Submitted by

Abhinav Raj

Department of Electrical and Electronics Engineering

Manipal Institute of Technology, Manipal

April 2026

# Acknowledgment

We would like to express our sincere gratitude to the Department of Electrical and Electronics Engineering at Manipal Institute of Technology, Manipal, for providing the necessary laboratory facilities and simulation tools to successfully complete this mini-project.

We thank our faculty members of the Analog System Design Lab for their guidance and technical support during the design and simulation of this temperature-sensing circuit.

# Abstract

This project presents the design and simulation of a thermistor-controlled LED circuit using an LM741 operational amplifier configured as a voltage comparator. The circuit uses an NTC (Negative Temperature Coefficient) thermistor in a voltage divider arrangement to produce a temperature-dependent voltage, which is compared against a fixed reference voltage by the LM741 comparator. When the sensed temperature exceeds a threshold — approximately 25 degrees Celsius — the comparator output switches from LOW to HIGH, turning on an LED through a current-limiting resistor.

The thermistor resistance is modelled using the Beta parameter equation with B = 3950 K and a nominal resistance of 10kΩ at 25 degrees Celsius. The simulation uses LTSpice with a parametric temperature sweep from 0 to 100 degrees Celsius in 5-degree steps, demonstrating the transition of the LED state across the full operating range. The circuit operates from a ±12V dual supply for the LM741 and a separate 5V supply for the sensing voltage divider.

# Table of Contents

| Chapter | Title | Page |
| --- | --- | --- |
| — | Acknowledgment | i |
| — | Abstract | ii |
| — | List of Figures and Tables | iii |
| 1 | Introduction | 1 |
| 2 | Background Theory | 2 |
| 2.1 | NTC Thermistor and Beta Model | 2 |
| 2.2 | Voltage Divider Sensing | 2 |
| 2.3 | LM741 as Voltage Comparator | 3 |
| 3 | Circuit Description | 4 |
| 3.1 | Circuit Schematic | 4 |
| 3.2 | Stage-by-Stage Description | 4 |
| 3.3 | Component Specifications | 5 |
| 4 | Calculations | 6 |
| 5 | Simulation Setup and Results | 8 |
| 6 | Conclusion and Contributions | 9 |
| — | References | 10 |

# List of Figures and Tables

| Item | Description |
| --- | --- |
| Figure 1 | LTSpice Schematic — Thermistor-Controlled LED Circuit |
| Table I | Component Specifications |
| Table II | Thermistor Resistance and Vtherm vs Temperature |
| Table III | LED Operating Conditions |
| Table IV | Simulation Parameters |
| Table V | Individual Contributions |

# Chapter 1: Introduction

Temperature sensing is a fundamental requirement in a wide range of electronic systems, from industrial process control and HVAC systems to medical devices and consumer electronics. While digital temperature sensors and microcontroller-based systems are common in modern designs, purely analog implementations using thermistors and operational amplifiers offer several advantages: they are instantaneous in response, require no firmware, consume minimal power, and are straightforward to design and calibrate.

This project implements one of the most fundamental analog temperature-sensing circuits: a thermistor voltage divider connected to an op-amp comparator that switches an LED ON or OFF based on whether the measured temperature is above or below a set threshold. The LM741, a classic general-purpose operational amplifier, is used in open-loop comparator configuration — exploiting its very high open-loop gain to produce a sharp digital-like output transition when the input differential crosses zero.

The NTC thermistor's resistance decreases as temperature increases, producing a rising voltage at the divider output as temperature rises. This voltage is compared against a fixed reference. When the thermistor voltage exceeds the reference, the comparator output swings to its positive rail, forward-biasing the LED through a current-limiting resistor and turning it on. This creates a simple but effective over-temperature indicator.

The simulation is performed in LTSpice using the built-in parametric temperature sweep directive (.step temp) combined with a transient analysis, with the thermistor resistance expressed as a mathematical function of the simulation temperature variable.

# Chapter 2: Background Theory

## 2.1 NTC Thermistor and Beta Parameter Model

A thermistor is a temperature-sensitive resistor. NTC (Negative Temperature Coefficient) thermistors decrease in resistance as temperature increases. The resistance-temperature relationship is described by the Beta parameter model:

R(T) = R0 * exp(B * (1/T - 1/T0))

Where:

- R(T) = thermistor resistance at temperature T (in Kelvin)

- R0 = nominal resistance at reference temperature T0 = 298.15 K (25 degrees Celsius) = 10,000 Omega

- B = Beta coefficient = 3950 K (material constant of the thermistor)

- T = operating temperature in Kelvin = T_Celsius + 273.15

- T0 = reference temperature = 298.15 K

In LTSpice, this is implemented directly as a resistor value expression using the built-in temp variable:

R2 = {10000 * exp(3950 * (1/(temp + 273.15) - 1/298.15))}

This expression evaluates the thermistor resistance at each step of the parametric temperature sweep, allowing the simulation to accurately model the thermistor's behavior across the full 0 to 100 degrees Celsius range.

## 2.2 Voltage Divider Sensing

The thermistor R2 and the fixed resistor R1 (10kΩ) form a voltage divider powered by a 5V supply (V3). The thermistor R2 is connected between V3 and the sensing node (Vtherm), while R1 connects from Vtherm to GND. The voltage at the Vtherm node is:

Vtherm = V3 * R1 / (R2 + R1) = 5 * 10k / (R_therm + 10k)

As temperature increases, R_therm decreases (NTC behaviour), so the denominator decreases and Vtherm increases. This means:

- At low temperature: R_therm is large, Vtherm is low

- At high temperature: R_therm is small, Vtherm is high

- At T = 25 degrees Celsius: R_therm = R1 = 10kΩ, so Vtherm = 5/2 = 2.5V (balanced divider)

## 2.3 LM741 as Voltage Comparator

The LM741 is a general-purpose operational amplifier with a very high open-loop voltage gain (typically 200,000 V/V or 106 dB). When used without feedback (open-loop), even the smallest differential input voltage drives the output to its saturation limits. This makes it ideal as a voltage comparator.

The comparator operation is as follows:

- Non-inverting input (+) receives Vtherm (temperature-dependent signal)

- Inverting input (-) receives a fixed reference voltage Vref

- When Vtherm > Vref: output saturates HIGH (approximately +10V on ±12V supply)

- When Vtherm < Vref: output saturates LOW (approximately -10V on ±12V supply)

The LM741 is powered by ±12V (V1 = +12V, V2 = -12V), giving output swing limits of approximately ±10V due to the internal circuitry headroom requirements. The output drives the LED through a current-limiting resistor R3.

Note: The LM741 is not a dedicated comparator and has a relatively slow slew rate (0.5 V/µs). For fast switching applications, a dedicated comparator such as the LM393 would be preferred. However, for slowly varying temperature signals, the LM741 is entirely adequate.

# Chapter 3: Circuit Description

## 3.1 Circuit Schematic

Figure 1: LTSpice Schematic — Thermistor-Controlled LED Circuit using LM741 Op-Amp Comparator

## 3.2 Stage-by-Stage Description

Stage 1 — Power Supply

The circuit uses a dual ±12V supply for the LM741 operational amplifier. V1 (+12V) provides the positive rail (VCC) and V2 (-12V) provides the negative rail (VEE). A separate 5V supply V3 powers the thermistor sensing voltage divider. Using a separate lower-voltage supply for the sensing stage improves the resolution of the voltage divider output (Vtherm ranges from approximately 1.1V to 4.7V across the 0 to 100 degree Celsius range).

Stage 2 — Thermistor Voltage Divider (R2, R1, V3)

R2 is the NTC thermistor whose resistance is modelled by the Beta equation. R1 (10kΩ) is a fixed pull-down resistor to GND. Together they form a voltage divider across the 5V supply. The output node Vtherm represents the temperature as a proportional voltage. At 25 degrees Celsius the two resistors are equal (both 10kΩ) producing exactly 2.5V at Vtherm. Below 25 degrees Celsius the thermistor resistance exceeds 10kΩ so Vtherm falls below 2.5V. Above 25 degrees Celsius the thermistor resistance falls below 10kΩ so Vtherm rises above 2.5V.

Stage 3 — LM741 Comparator (U1)

U1 (LM741) is configured in open-loop mode with no feedback. The non-inverting input (+) receives Vtherm. The inverting input (-) receives the reference voltage Vref = 2.5V. The comparator compares these two voltages and drives its output to +10V when Vtherm exceeds Vref, and to -10V when Vtherm is below Vref. The transition is extremely sharp due to the high open-loop gain of the LM741.

Stage 4 — LED Driver (R3, D1)

The output of U1 connects through current-limiting resistor R3 (330Ω) to the LED D1. When the comparator output is HIGH (+10V), current flows through R3 and D1, illuminating the LED. When the output is LOW (-10V), the LED is reverse-biased and off. R3 limits the LED current to a safe value and sets the brightness.

## 3.3 Component Specifications

| Component | Value / Type | Function |
| --- | --- | --- |
| V1 | +12V DC | Positive supply rail for LM741 (VCC) |
| V2 | -12V DC | Negative supply rail for LM741 (VEE) |
| V3 | +5V DC | Supply for thermistor voltage divider |
| R2 (Thermistor) | 10kΩ at 25°C, B=3950K, NTC | Temperature-sensitive resistor (upper arm of divider) |
| R1 | 10kΩ fixed | Lower arm of voltage divider (pull-down to GND) |
| U1 | LM741 Op-Amp | Open-loop voltage comparator |
| R3 | 330Ω | LED current-limiting resistor |
| D1 | LED (Vf = 2.0V, Ilimit = 50mA) | Visual output indicator |

# Chapter 4: Calculations

## 4.1 Thermistor Resistance at Key Temperatures

R(T) = 10000 * exp(3950 * (1/(T + 273.15) - 1/298.15))

| Temperature (°C) | T (Kelvin) | R_therm (Ω) | Vtherm (V) | LED State |
| --- | --- | --- | --- | --- |
| 0 | 273.15 | 33,621 | 1.146 | OFF |
| 10 | 283.15 | 20,175 | 1.657 | OFF |
| 20 | 293.15 | 12,535 | 2.219 | OFF |
| 25 | 298.15 | 10,000 | 2.500 | Threshold |
| 30 | 303.15 | 8,037 | 2.772 | ON |
| 40 | 313.15 | 5,302 | 3.268 | ON |
| 50 | 323.15 | 3,588 | 3.680 | ON |
| 60 | 333.15 | 2,486 | 4.004 | ON |
| 70 | 343.15 | 1,760 | 4.252 | ON |
| 80 | 353.15 | 1,270 | 4.436 | ON |
| 90 | 363.15 | 934 | 4.573 | ON |
| 100 | 373.15 | 698 | 4.674 | ON |

## 4.2 Switching Threshold Calculation

The LED turns ON when Vtherm > Vref = 2.5V. Solving for the threshold temperature:

Vtherm = 2.5V when R_therm = R1 = 10kΩ

10000 = 10000 * exp(3950 * (1/T - 1/298.15))

exp(3950 * (1/T - 1/298.15)) = 1

3950 * (1/T - 1/298.15) = 0

1/T = 1/298.15  =>  T = 298.15 K = 25 degrees Celsius

The switching threshold is exactly 25 degrees Celsius — the nominal temperature at which the thermistor equals R1.

## 4.3 LED Current When ON

When the comparator output is HIGH, the LM741 output swings to approximately +10V (2V below VCC due to internal headroom):

I_LED = (V_out - V_f) / R3 = (10V - 2.0V) / 330 Omega

I_LED = 8V / 330 Omega = 24.2 mA

This is within the LED model's current limit of 50mA. The LED operates safely.

## 4.4 Power Dissipation

P_R3 = I_LED^2 * R3 = (0.0242)^2 * 330 = 193 mW

P_LED = V_f * I_LED = 2.0 * 0.0242 = 48.5 mW

P_total_output_stage = P_R3 + P_LED = 193 + 48.5 = 241.5 mW

| Parameter | Value |
| --- | --- |
| Comparator output (HIGH) | ~+10V |
| LED forward voltage (Vf) | 2.0V |
| Current limiting resistor R3 | 330Ω |
| LED current | 24.2mA |
| Power in R3 | 193mW |
| Power in LED | 48.5mW |
| LED current limit (model) | 50mA — safe |

## 4.5 Voltage Divider Sensitivity

The sensitivity of Vtherm to temperature change at the threshold (25 degrees Celsius) can be estimated:

dVtherm/dT at T=25°C:

At 20°C: Vtherm = 2.219V

At 30°C: Vtherm = 2.772V

Sensitivity = (2.772 - 2.219) / (30 - 20) = 0.553V / 10°C = 55.3 mV/°C

This means a 1 degree Celsius change around the threshold produces approximately 55mV change in Vtherm — more than sufficient for a reliable comparator transition given the LM741's high open-loop gain.

# Chapter 5: Simulation Setup and Results

## 5.1 Simulation Directives

| Directive | Value | Purpose |
| --- | --- | --- |
| .step temp 0 100 5 | Sweep temperature from 0°C to 100°C in 5°C steps | Parametric temperature sweep (21 runs) |
| .tran 0 1m 0 startup | Transient analysis, 1ms duration | Observe steady-state LED state at each temperature |
| .option noopiter | Disable operating point iteration | Speed up simulation convergence |
| .model LED D(...) | Is=1e-21, N=1.8, Rs=5, Vfwd=2.0, Ilimit=0.05 | Custom LED diode model |
| .inc LM741.lib | Include LM741 SPICE model library | Accurate LM741 op-amp model |

## 5.2 Expected Simulation Results

The parametric sweep produces 21 simulation runs (0, 5, 10, 15, ... 100 degrees Celsius). The expected behaviour at each temperature is:

- 0 to 20 degrees Celsius: Vtherm < 2.5V, comparator output = LOW (~-10V), LED OFF (reverse biased)

- 25 degrees Celsius: Vtherm = 2.5V exactly, comparator at threshold — small noise determines state

- 30 to 100 degrees Celsius: Vtherm > 2.5V, comparator output = HIGH (~+10V), LED ON

The LED current of 24.2mA produces a clear visual indicator. The transition is sharp because the LM741's open-loop gain of ~200,000 V/V means even a 0.1mV differential at the input produces a 20V swing at the output (which then clips at ±10V).

## 5.3 Simulation Observations

- At temperatures below 25 degrees Celsius: Vtherm is below Vref, output stays at VEE (~-10V), D1 does not conduct, LED is OFF.

- At temperatures above 25 degrees Celsius: Vtherm rises above Vref, output switches to VCC (~+10V), current flows through R3 and D1, LED is ON.

- The switching occurs sharply at 25 degrees Celsius due to the high open-loop gain of the LM741.

- The 5V sensing supply (V3) limits Vtherm to the range 0V to 5V, ensuring the comparator inputs stay within the LM741 common-mode input range.

- Using a separate 5V supply for the sensing stage (rather than the ±12V supply) improves resolution — the full 5V range maps to the 0 to 100 degree Celsius span.

# Chapter 6: Conclusion and Contributions

## 6.1 Summary

This project demonstrated the design and simulation of a temperature-threshold switching circuit using a thermistor, a voltage divider, and an LM741 op-amp comparator. The circuit successfully implements a simple but effective over-temperature indicator: the LED remains OFF below 25 degrees Celsius and turns ON above 25 degrees Celsius. The switching threshold is set naturally by matching the fixed resistor R1 to the thermistor's nominal resistance R0 at the desired threshold temperature.

## 6.2 Conclusion

Key findings and design insights from this project:

- The Beta parameter model accurately represents NTC thermistor behaviour across the 0 to 100 degree Celsius range using a single parameter B = 3950K.

- Matching R1 = R0 = 10kΩ sets the switching threshold at exactly the thermistor's nominal temperature (25 degrees Celsius). This is a simple and reliable design rule.

- The LM741 in open-loop configuration provides sharp, reliable switching despite its relatively slow slew rate, because the temperature signal changes slowly.

- The 5V sensing supply combined with the 10kΩ voltage divider provides approximately 55mV per degree Celsius sensitivity near the threshold — well above the LM741 input offset voltage of ~1mV.

- R3 = 330Ω limits LED current to 24.2mA, which is safe and provides adequate brightness while staying within the LED model's 50mA limit.

This circuit forms the basis of many practical analog temperature control systems: thermostats, over-temperature protection circuits, fan controllers, and thermal alarms all use variations of this fundamental comparator-with-thermistor topology.

## 6.3 Individual Contributions

| Name | Roll No. | Contributions |
| --- | --- | --- |
| Abhinav Raj | 20 | LTSpice circuit design and simulation; thermistor Beta model implementation and verification |
| Anant Shankar Gupta | 09 | Hardware implementation on breadboard; component selection and testing |
| Nirja Parhi | 11 | Report writing and documentation; hardware implementation support |
| Shruti Mishra | 19 | Report writing, formatting, and final documentation |

# References

[1] Texas Instruments, "LM741 Operational Amplifier Datasheet," Texas Instruments, Revised 2015.

[2] Vishay Beyschlag, "NTC Thermistors — General Technical Information," Application Note, 2008.

[3] P. Horowitz and W. Hill, The Art of Electronics, 3rd ed. Cambridge University Press, 2015, pp. 222–226.

[4] S. Sedra and K. C. Smith, Microelectronic Circuits, 7th ed. Oxford University Press, 2015.

[5] Linear Technology, "LTSpice XVII User Guide," Analog Devices, 2023.

# Annexure A: LTSpice Simulation Directives

The following directives were used in the LTSpice simulation file (Thermistor_controlled_LED_using_LM741.asc):

| Directive / Parameter | Value | Notes |
| --- | --- | --- |
| Simulation type | .tran 0 1m 0 startup | 1ms transient with startup bias point |
| Temperature sweep | .step temp 0 100 5 | 21 steps from 0°C to 100°C |
| Convergence option | .option noopiter | Faster convergence for stepped runs |
| Thermistor model (R2) | {10000 * exp(3950 * (1/(temp+273.15) - 1/298.15))} | Beta equation with LTSpice temp variable |
| LED model | .model LED D(Is=1e-21 N=1.8 Rs=5 Vfwd=2.0 Ilimit=0.05) | Custom SPICE diode model |
| Op-amp model | .inc LM741.lib | External LM741 SPICE library |
| V1 (VCC) | 12V | Positive supply |
| V2 (VEE) | -12V | Negative supply |
| V3 (Sensing supply) | 5V | Thermistor divider supply |